ISP Network Observability Platform

Operations Manual

Day-to-day running, alert handling, maintenance and recovery for the monitoring server hosted on a Linux machine inside the ISP's management network.

Audience: the engineer who owns the platform. Management receive the separate Management Overview. Companion documents in the repository docs/ folder: ARCHITECTURE, INSTALL, SETUP-INPUTS, GO-LIVE, SECURITY, snmpv3, deadman, whatsapp, server-monitoring, failure-drills.

Engineer edition - detailed, internal  ·  October 2026

# Contents

1. What the platform is

2. Daily routine for the NOC

3. Reading and acting on alerts

4. Alert channels: Telegram, WhatsApp and the dead-man's switch

5. Routine changes

6. Server health and troubleshooting

7. Backup, upgrade and recovery

8. Security operations

9. Command cheat sheet

10. Go-live checklist

Appendix A: ports, files and secrets

Appendix B: server sizing

# 1. What the platform is

A self-hosted monitoring stack that watches PTP links, MikroTik routers, PtMP sectors with their CPEs, and PPPoE subscribers. It runs in Docker on one Linux server inside the ISP's own management network. It only reads from network devices and only calls out to Telegram, the heartbeat service and (optionally) WhatsApp; nothing is exposed to the internet.

| Component | Role |
| --- | --- |
| Prometheus (:9090) | Collects metrics, evaluates alert rules, stores 90 days of history (20 GB cap). |
| Grafana (:3000) | Dashboards. NOC staff log in as Viewers. |
| Alertmanager (:9093) | De-duplicates and routes alerts to Telegram, WhatsApp and the heartbeat; holds silences. |
| snmp-exporter / blackbox-exporter | Poll devices over SNMP (v1/v2c or encrypted v3) and ping them on Prometheus's behalf. |
| routeros-exporter | Reads subscriber sessions and topology from MikroTik routers via the RouterOS API. |
| node-exporter | Reports the monitoring server's own CPU, memory, disk and clock. |
| whatsapp-bridge | Turns phone-worthy critical alerts into capped WhatsApp template messages (dry-run by default). |

Everything is configuration-as-code in the git repository; nothing important is configured only in a web UI. Secrets live in the gitignored secrets/ folder.

# 2. Daily routine for the NOC

- Open the NOC Overview dashboard. The Active Incidents table lists every firing alert, one row per object, longest-running first. An empty table is the healthy state.

- Glance at the state counts. Links, routers and sectors up or down; click a row to open its detail dashboard.

- Check Server Health once a day. Disk-free trend, memory, component up/down, notification failures and the WhatsApp budget.

- Check the heartbeat. The healthchecks.io check should be green. If it goes red the whole stack has gone silent (section 4).

- Check Prometheus Targets weekly. http://<server>:9090/targets: every job should be UP. A new device that is DOWN usually means a wrong IP, SNMP credential or firewall rule.

- Read the 08:00 capacity digest. Sectors over their client or throughput budget arrive as one Telegram message each morning (08:00 Africa/Lagos).

| Dashboard | Use it to |
| --- | --- |
| NOC Overview | See what is wrong right now and for how long. |
| PTP Link Monitoring | Investigate one backhaul link: latency, loss, signal, noise, capacity. |
| MikroTik Fleet Overview / Router Detail | Router reachability, CPU, memory, interface traffic. |
| WAN/LAN Interfaces | Each router's WAN uplink against its LAN side. |
| Sector Fleet Overview / Sector Detail | Sector load, noise, worst CPEs; drill into one sector and its CPEs. |
| Subscriber Overview | Online vs provisioned sessions per POP, flappers, disconnect reasons. |
| Capacity Planning | 24-hour peak, busy-hour and average traffic for planning upgrades. |
| Server Health | The monitoring server and the stack's own health. |

# 3. Reading and acting on alerts

One failure produces one message. The header names the object and the worst severity; the lines below list each alert on it. Symptoms are suppressed while their cause is firing (a down link silences its signal, loss and error alerts), so the first message is the cause.

| Severity | Meaning | Expected response |
| --- | --- | --- |
| Critical (red) | Something is down or about to stop working. | Act now. Repeats every 2 hours on Telegram until fixed. |
| Warning (orange) | Degraded but working. | Look within the working day. Repeats every 12 hours; no 'resolved' message. |
| Capacity digest | A sector is over its budget. | Plan capacity. Delivered once daily at 08:00. |

## Common alerts and the first thing to do

| Alert | What it means | First checks |
| --- | --- | --- |
| LinkDown / LinkDownProlonged | A PTP link is unreachable (Telegram at 5 min; WhatsApp page only at 30 min). | Is either end powered? If a whole POP is affected, check the POP router. |
| WeakSignal / LowLinkQuality / PacketLoss / HighLatency | Link up but degraded for most of the last 30 minutes. | Look at the link trend; suspect alignment, interference or weather. |
| CoreRouterDown | A router does not answer SNMP. | Ping it; check power and uplink. If ExporterDown or MonitoringVantageLost also fires, the router is probably fine. |
| SectorDown / sector CPE alerts | A sector AP or its clients are affected. | Check the sector's management route and power; open Sector Detail. |
| SubscriberMassOutagePOP / Router | Online subscribers dropped sharply on a POP or router. | Check the POP router and backhaul first. |
| Anomaly alerts | A metric deviates from its own recent baseline. | Investigate slow bleeds: eroding subscriber counts, CPU or memory creep. |
| MonitoringVantageLost | The monitoring server has no network access. | Someone may be working on the server. Per-device alerts are held back until it recovers. |
| ExporterDown | A collector or stack component stopped. | docker compose ps; docker compose logs <name>. Devices behind it are probably fine. |
| ServerDiskLow / ServerDiskFillingUp | The server's disk is nearly full, or will be within 24 hours. | Free space or extend the volume before Prometheus stops ingesting. |
| PrometheusRuleFailures / AlertDeliveryFailing | The alert pipeline itself is impaired. | Check Prometheus and Alertmanager logs and the channel credentials. |
| healthchecks.io 'down' (outside the stack) | No heartbeat for 10 minutes: server, Docker, Prometheus, Alertmanager or the internet uplink is dead. | Check the server is powered and online; docker compose ps. This is the only alert that works when the server is dead. |

# 4. Alert channels: Telegram, WhatsApp and the dead-man's switch

## Telegram

The NOC group receives every alert. Warnings use a separate receiver that sends no 'resolved' message. The bot token is a Docker secret (secrets/telegram_bot_token.txt). The group id is set on one line of alertmanager/alertmanager.yml; both receivers reuse it.

To change the Telegram group (full steps: SETUP-INPUTS.md, section D):

- Add the bot to the new group.

- Send a message in the group, then open https://api.telegram.org/bot<token>/getUpdates and read "chat":{"id":...} (group ids are negative, e.g. -100...).

- In alertmanager/alertmanager.yml, search for noc_chat_id and replace the number on that line.

- Validate with amtool (section 9), then docker compose restart alertmanager.

To change the bot: put the new token in secrets/telegram_bot_token.txt and restart Alertmanager.

## WhatsApp (optional, cost-controlled)

WhatsApp is the only paid channel, so it is restricted by design:

- Phone-worthy criticals only. Rules labelled page="true" (link down 30+ minutes, core router down, sector down, subscriber mass outage). Everything else stays on Telegram.

- Weather-proof. A link down for 5 minutes alerts Telegram; WhatsApp fires only if it is still down after 30 minutes (LinkDownProlonged, tunable in prometheus/rules/link-alerts.yml).

- Dry-run first. WHATSAPP_MODE=dry_run logs what would be sent and sends nothing.

- Hard caps. WHATSAPP_DAILY_CAP (default 10) and WHATSAPP_MONTHLY_CAP (default 100) per recipient; extra sends are dropped, never queued.

- Budget warnings at 80% of a cap and when sends fail. Grouping: a wide outage is one message per scope, at most every 6 hours.

Go-live steps:

- Meta developer app with the WhatsApp product; note the Phone Number ID; create a System User token.

- Get the Utility template isp_critical_alert approved (header 'ISP Network Alert'; body 'Critical: {{1}}. Details: {{2}} Check Grafana for the full picture.'; footer 'Automated alert. Replies are not monitored.'; static 'Open in Telegram' button).

- Token in secrets/whatsapp_access_token.txt; WHATSAPP_PHONE_NUMBER_ID and WHATSAPP_RECIPIENTS (digits only, country code, no '+') in .env. Start with one recipient.

- docker compose up -d --build whatsapp-bridge, trigger a test critical, confirm 'DRY_RUN would send' in its logs.

- Set WHATSAPP_MODE=live in .env; docker compose up -d whatsapp-bridge.

Cost: Meta bills per message per recipient by template category and country. Worst case = monthly cap x Meta's current rate.

## Dead-man's switch (heartbeat)

Nothing inside the stack can report the stack's own death. The Watchdog alert is always firing; Alertmanager re-sends it every minute to a healthchecks.io check (URL in secrets/deadman_url.txt). When the pings stop for 10 minutes (5-minute period + 5-minute grace), healthchecks.io notifies the NOC through its own channels (Telegram is configured). Watchdog never reaches Telegram or WhatsApp directly and does not appear on the incident board.

- Not the gateway check. vantage-points.yml (the POP router) tells whether the server can see the network; the heartbeat tells whether the server is alive at all.

- Planned maintenance: pause the check on healthchecks.io first, unpause after.

- Test: docker compose stop alertmanager; the check should go red within ~10 minutes. Or send a failure signal directly (section 9).

- Fresh install without a check: set the Watchdog route's receiver to deadman-off, or the placeholder URL fails every minute and trips AlertDeliveryFailing.

# 5. Routine changes

After editing any file below, validate before applying (section 9).

## Add a router

- Add its IP and labels (site, role, router, wan_ifname, and auth if it uses SNMPv3) to prometheus/targets/mikrotik-routers.yml. Reloaded within a minute.

- For subscriber data: an entry in routeros_exporter/config.yml (name = the router label) and a read-only API user in secrets/routeros_api_credentials.json under the same name; then docker compose restart routeros-exporter.

- On the router: SNMP (preferably a v3 user) and the API service, both limited to the server's IP.

## Add a PTP link or a sector

- Links: prometheus/targets/links.yml (endpoints, module and auth). Sectors: prometheus/targets/mikrotik-sectors.yml.

- Sectors (bridged L2, management IP in the POP's /25) are reachable only via routes on the sector and the POP router. Today the POP masquerade rule works around this but hides the server's address. Use a NARROW route on the sector (monitoring subnet only, no default route), SNMPv3, an access list, and a POP rule dropping customer -> management. No subscriber impact. Commands below.

- Confirm the new target is UP at /targets, then in the dashboards.

# on the sector: narrow route to the monitoring network only (no default route)

/ip route add dst-address=<monitoring-server-subnet> gateway=<POP-router-IP-in-this-/25> comment="mgmt -> NMS"

# on the sector: SNMPv3 read-only user, allowed from the monitoring server only

/snmp community add name=monitoring security=private authentication-protocol=SHA1 \

authentication-password="<auth-pass>" encryption-protocol=AES encryption-password="<priv-pass>" \

addresses=<monitoring-server-ip>/32 read-access=yes write-access=no

/snmp set enabled=yes contact="NOC" location="<POP> - <sector>"

# on the sector: management services only from management addresses (customers share its L2 bridge)

/ip service set [find where name~"winbox|ssh|api|www"] address=<mgmt-ranges>

# on the POP router: route for the sector management range (if not connected)

/ip route add dst-address=<sector-mgmt-range> gateway=<next-hop> comment="mgmt -> sectors"

# on the POP router: PPP customers must never be forwarded to management addresses

/ip firewall filter add chain=forward src-address=<ppp-pool-ranges> dst-address=<mgmt-ranges> action=drop comment="no ppp customer -> mgmt"

# then remove the POP masquerade rule used to reach the sectors

# check: RouterOS, then from the server (1 = working)

/ip route print ; /ping <monitoring-server-ip> count=3

curl -s 'http://localhost:9116/snmp?target=<sector-ip>&module=mikrotik_sector&auth=mikrotik_v3' | grep -c '^sysUpTime'

# proof, from a PPPoE customer: the POP router still answers ping, the sector/CPE management IPs do NOT

Why this adds no customer exposure: PPPoE customers have no address in the management range and are routed only by the POP router. The routes carry management traffic; the forward drop stops the POP router from passing PPP traffic onto the new routes. Keep the sector route narrow (monitoring subnet, not 0.0.0.0/0) and no routing protocols on sectors.

## Move a device to SNMPv3

ISP policy: SNMPv3 only. Full guide: docs/snmpv3.md. v3 users live in secrets/snmp_auths.yml (gitignored), loaded next to snmp_exporter/snmp.yml; a target selects one with its auth label.

- Create a read-only v3 user on the device, limited to the server's IP. Keep the v2c community until the switch is confirmed.

- Fill in the matching entry in secrets/snmp_auths.yml; docker compose restart snmp-exporter.

- Test with the curl command in section 9; 1 means it works.

- Set auth: <name> on the target; confirm UP at /targets.

- Remove the v2c community from the device.

Spelling matters: snmp-exporter wants SHA (not SHA1) and AES256 (not AES-256). A wrong spelling or a duplicate auth name stops snmp-exporter at startup.

| Device | SNMP (25 Oct 2026) |
| --- | --- |
| pop1-home core router | v3 mikrotik_v3 (SHA / AES-128); v2c off |
| SAF POP1-POP7 pair | v3 saf_v3 (SHA-256 / AES-256); v2c off |
| AirFiber POP6-POP2 | v2c - firmware has no SNMPv3; violates the v3-only policy, management decision pending (ping-only / documented exception / replace) |
| Other routers, sector 5B, airMAX LiteBeams | v2c - to migrate (sectors together with their routes) |

## Change what 'the server's own network' means

Edit prometheus/targets/vantage-points.yml: the production server's default gateway (the POP router) and optionally a public address. MonitoringVantageLost fires only when every listed target is unreachable.

## Change a threshold or add an alert

- Edit the rule under prometheus/rules/, update its test in prometheus/rules/tests/, validate, then docker compose restart prometheus.

- Degradation alerts use a sustained-breach pattern (breaching now and in at least 80% of the last 30 minutes); keep it so one bad scrape does not alert.

## Silence alerts during planned work

- Alertmanager UI (http://<server>:9093) > Silences > New Silence. Match on labels (link_id, router) and set an end time.

- Silence only the objects being worked on. If the server itself is being worked on, also pause the heartbeat check.

# 6. Server health and troubleshooting

| Symptom | Likely cause | What to do |
| --- | --- | --- |
| Many devices 'down' at once | A collector stopped, or the server lost its own network. | Look for ExporterDown or MonitoringVantageLost; fix that first. |
| No alert messages at all | Alertmanager or Telegram problem. | docker compose logs alertmanager; check the bot token and noc_chat_id; AlertDeliveryFailing; Notifications panel on Server Health. |
| Alertmanager runs but sends nothing on a new Linux server | Secret files are 600, unreadable to the non-root containers. | chmod 700 secrets && chmod 644 secrets/*, then docker compose up -d. |
| A target is DOWN in /targets | Wrong IP, SNMP credential, firewall, or the device is off. | Ping it from the server; check the community/v3 user and the device's source-IP restriction. |
| SNMPv3 target times out | Username, password, algorithm or allowed address differs from the device (RouterOS drops unknown users silently). | Re-check the device's v3 user against secrets/snmp_auths.yml; test one request at a time. |
| snmp-exporter will not start | Bad spelling (SHA1, AES-256) or a duplicate auth name. | docker compose logs snmp-exporter shows the error; fix secrets/snmp_auths.yml. |
| Heartbeat check red | Server, Docker or uplink down; or Alertmanager stopped. | Check the server; docker compose ps; docker compose logs alertmanager. |
| Grafana panels empty | Prometheus not scraping or restarted. | Open /targets; docker compose restart prometheus; check disk space. |
| routeros-exporter shows no data | Missing credentials or API blocked. | docker compose logs routeros-exporter; check the credentials entry and API reachability. |
| Disk almost full | Prometheus history growing. | Free space, or lower storage.tsdb.retention (time/size) in prometheus/prometheus.yml. |
| WhatsApp message not received | Dry-run, cap reached, template not approved, token expired. | docker compose logs whatsapp-bridge; check mode, budget panel and template status. |

Logs: docker compose logs --tail 100 <service>. Services: prometheus, alertmanager, grafana, snmp-exporter, blackbox-exporter, routeros-exporter, node-exporter, whatsapp-bridge.

# 7. Backup, upgrade and recovery

## What to back up

| Item | Where | How |
| --- | --- | --- |
| Configuration (rules, targets, dashboards, compose) | The git repository | git push to the remote. The source of truth. |
| Secrets | secrets/ (Grafana, Telegram, RouterOS, WhatsApp, SNMPv3 users, heartbeat URL), .env, routeros_exporter/config.yml | Encrypted copy kept apart from the repository. |
| Metric history | Docker volume prometheus_data | Optional; stop Prometheus briefly and archive the volume. |
| Grafana users and preferences | Docker volume grafana_data | Optional; dashboards are in git. |
| Silences / WhatsApp counters | alertmanager_data, whatsapp_bridge_data | Optional. |

docker volume ls | grep -E "prometheus_data|grafana_data"   # exact names

docker run --rm -v <prometheus_volume>:/d -v "$PWD":/b alpine tar czf /b/prometheus_data.tgz -C /d .

## Upgrade

git pull

docker compose config -q

docker compose up -d --build

docker compose ps        # all Up; then check /targets

Rules, scrape config and retention are baked into the published Prometheus image (current: chikere/isp-observability:v1.4). After changing them, build and push a new tag before using docker-compose.image.yml (DOCKER.md).

## Move to another computer

Use git for configuration; copy the secrets separately over an encrypted channel; restore secret permissions (chmod 700 secrets && chmod 644 secrets/*). Stop the old stack first so two machines never alert. Full steps: INSTALL.md, 'Moving an existing installation'.

## Recover a dead server

- Install Docker on the replacement Linux machine (INSTALL.md).

- git clone the repository; restore secrets/ and .env (or run scripts/init.sh and re-enter them per SETUP-INPUTS.md).

- docker compose up -d --build. History is lost unless the volume archive is restored; configuration and alerts come back from git. The heartbeat turns green again on its own.

# 8. Security operations

- Rotate credentials when staff leave or a token is exposed: Grafana, Telegram bot (@BotFather), RouterOS API users, SNMPv3 users (device and secrets/snmp_auths.yml), WhatsApp token, heartbeat URL (regenerate the check's URL). Update the file and docker compose up -d <service>. Grafana reads its password file only on first start: change an existing password in the UI or with docker compose exec grafana grafana cli admin reset-admin-password '<new>'.

- Firewall. Only NOC workstations reach Grafana (3000); 9090/9093 only engineers. The exporter ports (9100, 9115, 9116, 9436) are bound to 127.0.0.1 in docker-compose.yml and unreachable from the LAN - keep it that way.

- Read-only access. The RouterOS monitoring user stays in group read; SNMP communities and v3 users are limited to the server's IP.

- SNMPv3 only (ISP policy). authPriv on every device; remove the v2c community after switching. AirFiber cannot comply - follow the management decision; until then restrict it to the server's IP.

- Sector routes stay narrow. Monitoring-subnet route only on sectors (no default route), no routing protocols on sectors, POP rule dropping customer -> management; test from a customer address after every POP.

- Secrets on disk. secrets/ folder 700, files 644 (containers read them as non-root users).

- Dedicated server. No other services; SSH keys only; OS and Docker updates on a schedule. Management network only, no leg in the customer network (SECURITY.md checklist).

# 9. Command cheat sheet

# status and logs

docker compose ps

docker compose logs --tail 100 -f <service>

# validate before applying (no local promtool: run it in Docker)

docker run --rm -v "$PWD/prometheus:/p" --entrypoint sh prom/prometheus:v3.13.2 \

-c 'cd /p && promtool check rules rules/*.yml && promtool test rules rules/tests/*.yml'

docker run --rm -v "$PWD/alertmanager:/a" --entrypoint amtool prom/alertmanager:v0.33.1 \

check-config /a/alertmanager.yml

docker compose config -q

# apply

docker compose up -d --build

docker compose restart prometheus        # rules / scrape / retention

docker compose restart alertmanager      # routing, chat id, heartbeat

docker compose restart snmp-exporter     # snmp.yml or secrets/snmp_auths.yml

docker compose restart routeros-exporter # its config or credentials

# SNMP test against one device (1 = working)

curl -s 'http://localhost:9116/snmp?target=<ip>&module=mikrotik&auth=mikrotik_v3' | grep -c '^sysUpTime'

# heartbeat: send a test failure, then recover

curl -fsS -X POST "$(cat secrets/deadman_url.txt)/fail"

curl -fsS -X POST "$(cat secrets/deadman_url.txt)"

# Grafana admin password

docker compose exec grafana grafana cli admin reset-admin-password '<new>'

# secret permissions on Linux

chmod 700 secrets && chmod 644 secrets/*

# WhatsApp bridge

docker compose logs --tail 50 whatsapp-bridge

docker compose up -d whatsapp-bridge     # after editing .env

# 10. Go-live checklist

Work through docs/GO-LIVE.md. Status as of October 2026:

| Item | Status |
| --- | --- |
| SNMPv3 only (ISP policy) | Done on pop1-home and the SAF pair; sectors/routers next; AirFiber decision pending |
| Sector routes (narrow), SNMPv3, customer -> mgmt drop, masquerade removed | Pending - pilot POP first |
| Dead-man's switch (healthchecks.io + Telegram) | Done and tested; pause until production runs |
| Strong Grafana password | Done |
| Telegram production group (noc_chat_id) | Done |
| Image with current rules | Done: v1.4 published |
| Production server (Appendix B), Docker at boot, firewall | Pending - server not installed |
| Production gateway in vantage-points.yml | Pending - needs the production gateway IP |
| Replace lab devices in the target files | Pending - with the production server |
| Raise retention size cap 20 GB -> ~250 GB | Pending - at production setup |
| Secrets 700/644 on the Linux server | Pending - scripts/init.sh sets it |
| WhatsApp production number, billing, template | Optional - stay in dry-run for the first week |
| Validate, all targets UP, one test alert, failure drill | Pending - at cut-over |
| Backups of secrets; config pushed to git; named owner | Pending - at cut-over |

# Appendix A: ports, files and secrets

| Port | Service | Who reaches it |
| --- | --- | --- |
| 3000 | Grafana | NOC workstations |
| 9090 | Prometheus | Engineers |
| 9093 | Alertmanager | Engineers |
| 9100 / 9115 / 9116 / 9436 | node / blackbox / snmp / routeros exporters | Bound to 127.0.0.1: the server only |
| 8080 (internal) | whatsapp-bridge | Alertmanager only (not published) |

| File | Purpose |
| --- | --- |
| docker-compose.yml | All services, volumes, secrets, port bindings. |
| prometheus/prometheus.yml | Scrape jobs and retention (90d / 20 GB). |
| prometheus/targets/*.yml | Routers, sectors, links, vantage points, each with its SNMP auth (reload within a minute). |
| prometheus/rules/*.yml, rules/tests/ | Alert and recording rules (55 alerts, incl. Watchdog) and their tests. |
| alertmanager/alertmanager.yml | Routing, inhibition, noc_chat_id, WhatsApp and Watchdog routes. |
| snmp_exporter/snmp.yml | SNMP modules and v1/v2c communities. |
| grafana/dashboards/*.json | Dashboards (auto-provisioned). |
| routeros_exporter/config.yml | Routers polled over the RouterOS API (gitignored). |
| .env | WhatsApp settings (gitignored). |
| docs/src/build-docx.js | Source of this manual and the Management Overview: npm run build. |

| Secret (secrets/, gitignored) | Holds |
| --- | --- |
| grafana_admin_password.txt | Grafana admin password (first start only) |
| telegram_bot_token.txt | Telegram bot token |
| routeros_api_credentials.json | Read-only RouterOS API users, keyed by router name |
| snmp_auths.yml | SNMPv3 users and passwords |
| deadman_url.txt | healthchecks.io ping URL |
| whatsapp_access_token.txt | Meta access token |

# Appendix B: server sizing

Estimate for a 10,000-subscriber ISP, from measured series counts: about 12 series per subscriber (RouterOS API), 25 per wireless CPE (sector registration table plus derived baselines), plus PPPoE interfaces in SNMP and their reconnect churn. Total roughly 450-500k active series, ~17,000 samples per second, ~2 GB of history per day.

|  | Minimum | Recommended (2x headroom) |
| --- | --- | --- |
| CPU | 4 cores | 8 cores |
| RAM | 16 GB | 32 GB |
| Disk | 500 GB SSD | 1 TB NVMe SSD |
| OS | Ubuntu Server 24.04 LTS / Debian 12 | same |
| Other | 1 Gbps NIC on the management network; UPS | + RAID 1 or scheduled volume backup |

At this scale raise the retention size cap to about 250 GB (prometheus/prometheus.yml). After a week in production, confirm with prometheus_tsdb_head_series, rate(prometheus_tsdb_head_samples_appended_total[1h]) and prometheus_tsdb_storage_blocks_bytes. Windows hosts are not recommended for production (INSTALL.md section 2); use Linux, or an Ubuntu VM on the Windows machine.