ISP Network Observability Platform

Management Overview

What the platform does, the technology behind it, how sectors are made reachable, how the team is alerted and responds, how it is secured, what it costs, and where the rollout stands.

Audience: management and stakeholders.

Management edition  ·  October 2026

Contents

1. Summary

2. Technology behind the platform

3. What is monitored

4. Sector Reachability

5. Alert channels

6. Reading and acting on alerts

7. What the NOC sees

8. Server health and troubleshooting

9. Security

10. Reliability

11. Running costs and hardware

12. Rollout status

13. What the business provides

14. Known limitations

15. Possible next phases

16. Ownership

1. Summary

The ISP Network Observability Platform watches the whole network - core routers, backhaul links, wireless sectors and their customer radios, and subscriber sessions - and tells the NOC about problems within minutes, usually before customers call.

- One problem, one message. When a link or router fails, the team gets a single message naming the cause, not a flood of side-effects.

- Fewer false alarms. Short weather fades and one-off glitches do not page anyone; alerts need a sustained problem.

- Subscriber-level visibility. Online vs provisioned customers per POP and router, mass-outage detection, and disconnect reasons.

- Capacity planning. A daily 08:00 digest of overloaded sectors and busy-hour trends for upgrade decisions.

- Self-hosted and low cost. One server inside the ISP's own network, free open-source software; the only optional paid item is WhatsApp paging, which is capped.

| At a glance |  |
| --- | --- |
| Alert rules | 55, checked by 34 automated test scenarios |
| Dashboards | 10 (NOC overview, links, routers, sectors, subscribers, capacity, server health) |
| Alert channels | Telegram (all alerts), WhatsApp (phone-worthy only, optional), independent heartbeat |
| Device polling | SNMPv3 only (ISP policy) - live on the core router and SAF links; RouterOS API; ping |
| History kept | 90 days |
| Access to network devices | Read-only |

2. Technology behind the platform

Built entirely from established open-source monitoring components, each running in its own Docker container on one Linux server. Every component is pinned to a tested version and never auto-updates.

| Component | Technology (version) | What it does |
| --- | --- | --- |
| Metrics and alert engine | Prometheus 3.13 | Collects every measurement, evaluates the 55 alert rules, keeps 90 days of history. |
| Dashboards | Grafana 13.2 | The NOC's screens: overview, links, routers, sectors, subscribers, capacity, server health. |
| Alert routing | Alertmanager 0.33 | Groups related alerts into one message, suppresses symptoms, sends to Telegram / WhatsApp / heartbeat. |
| SNMP poller | Prometheus SNMP Exporter 0.30 | Reads routers and radios over SNMP (v3, v2c or v1 per device). |
| Reachability prober | Prometheus Blackbox Exporter 0.28 | Pings links and the server's own gateway. |
| Subscriber collector | In-house RouterOS API collector (Python) | Reads PPPoE sessions, provisioned users, disconnect reasons and topology from MikroTik routers. |
| Server monitor | Prometheus Node Exporter 1.9 | The monitoring server's own CPU, memory, disk and clock. |
| WhatsApp gateway | In-house bridge to Meta WhatsApp Cloud API | Turns phone-worthy criticals into capped template messages. |
| Heartbeat | healthchecks.io (external) | Alerts the team if the platform itself goes silent. |
| Runtime | Docker Engine + Compose on Ubuntu Server LTS | Runs and restarts all of the above. |

How devices are read

Policy (set by the ISP): SNMPv3 only. SNMPv1/v2c send their access string in clear text with no authentication, so they are being retired. SNMPv3 authPriv proves each request comes from the monitoring server and encrypts it.

| Device family | Status against the SNMPv3-only policy |
| --- | --- |
| MikroTik core router | Compliant - SNMPv3 authPriv (SHA / AES-128); v2c switched off |
| SAF backhaul radios | Compliant - SNMPv3 authPriv (SHA-256 / AES-256); v2c switched off |
| MikroTik sectors | To migrate - SNMPv3 is configured together with the sector routes (section 4) |
| Other MikroTik routers | To migrate - same method as the core router |
| Ubiquiti airMAX | To confirm - depends on the firmware's SNMPv3 support |
| Ubiquiti AirFiber | Cannot comply - the firmware has no SNMPv3. Needs a decision (below) |
| Subscriber data (MikroTik) | RouterOS API with a read-only user (not SNMP) |
| Link reachability | ICMP ping (not SNMP) |

Decision needed: AirFiber

| Option | Effect |
| --- | --- |
| A. Ping-only monitoring | Fully policy-compliant. Up/down and latency stay; signal, capacity and error metrics for AirFiber links are lost. |
| B. Documented exception | Keep SNMPv2c on AirFiber only, read-only, accepted solely from the monitoring server on the management network. Full metrics; a known, contained deviation from policy. |
| C. Replace or upgrade | Move those links to SNMPv3-capable radios over time; option A or B until then. |

3. What is monitored

| Area | What the platform tracks |
| --- | --- |
| Core routers (MikroTik) | Reachability, CPU, memory, uptime, WAN uplink and interface traffic. |
| Backhaul links (Ubiquiti airMAX / AirFiber, SAF) | Up/down, latency, packet loss, signal, noise, link capacity, errors. |
| Wireless sectors and customer radios | Clients per sector, sector load and noise, each customer radio's signal and its drift over 7 days, sector-wide degradation. |
| Subscribers (PPPoE) | Online vs provisioned per POP and router, sudden drops (mass outage), repeated reconnects, authentication failure spikes. |
| POP dependencies | Knows which sectors and routers sit behind each POP, so a POP outage is reported once as the root cause. |
| The monitoring server itself | Disk, memory, CPU, clock, and the health of every platform component. |

4. Sector reachability

A sector can only be monitored if the monitoring server can reach its management address and the sector can reply. This is the one network change the rollout needs.

How a customer is connected today

| Hop | Role |
| --- | --- |
| POP router (MikroTik) | PPPoE server; holds the PPP secrets for that POP and routes every customer's traffic. |
| Sector (MikroTik AP) | Layer-2 bridge between the POP router and the client radios; one management IP. |
| Client radio (CPE) | Layer-2 bridge to the customer's premises. |
| Customer home router (MikroTik) | Radio on ether1; PPPoE client dials the POP router; the home network reaches the internet through that PPP session with NAT. |

Customers therefore have no address in the management network: everything they send travels inside their PPP session and is routed by the POP router. In practice a customer can ping the POP router but cannot ping the radios - the management side is already separated from customers.

| Topic | Detail |
| --- | --- |
| The bottleneck | The sector's management IP is not reachable from the monitoring server: there is no IP route between them in either direction, so ping, SNMP, Winbox and NTP from the server cannot get there or come back. |
| Today's workaround | A masquerade (NAT) rule on the POP router makes the server's requests appear to come from the POP router. It works, but every sector sees the POP router instead of the monitoring server, so a sector cannot restrict SNMP to the monitoring server's own address. |
| The solution | A management IP and route on both the POP router and the sector: the POP router routes the sector management range, and each sector gets a narrow route back to the monitoring network (not a default route). The masquerade rule is then removed and the server sees each sector at its real address. |
| Subscriber impact | None. Customer traffic is PPPoE bridged through the sector at Layer 2 and never uses its routing table; the route carries only traffic the sector itself sends. Safe to apply during business hours. |
| Effort | Per sector: management IP, one route, SNMPv3 user. Per POP router: the matching route and one firewall rule. Verified from the server before moving on. |

Security implications

The change adds no new exposure for customers, on one condition that is simple to meet and to prove:

- Why it is safe. Customers reach anything only through their PPP session on the POP router, and the routes carry management traffic only. Sectors keep bridging customer traffic exactly as today.

- The one condition. Once the POP router has a route to the sector management range it would, by default, also forward PPP customer traffic there. One firewall rule on the POP router - drop traffic from PPP customer addresses to the management range - keeps the management side exactly as unreachable as it is today.

- The proof. The test already used today, repeated after the change at each POP: from a customer, the POP router still answers and the radios still do not.

- Narrow and read-only. The sector's route points only at the monitoring network; no routing protocols run on sectors; SNMPv3 only, read-only; no management address is exposed to the internet.

Recommended rollout: one POP first (management IP and routes, SNMPv3, POP firewall rule, customer-side ping test), review, then POP by POP.

5. Alert channels

| Channel | Receives | Why |
| --- | --- | --- |
| Telegram (NOC group) | Every alert, grouped per affected link, router, sector or POP. Criticals also send a 'resolved' message. | Free, instant, shared by the whole NOC. The primary channel. |
| WhatsApp (optional) | Only phone-worthy criticals: link down 30+ minutes, core router down, sector down, subscriber mass outage. | Wakes the on-call engineer for real outages only. |
| Daily digest (08:00) | Sectors over their client or throughput budget. | Capacity issues are planning items, not emergencies. |
| Independent heartbeat | A Telegram message from an outside service if the platform itself goes silent for 10 minutes. | Covers server, power or internet failure, which nothing on the server can report. |

WhatsApp cost controls

- Weather-proof. A link down for 5 minutes alerts Telegram only; WhatsApp fires only if it is still down after 30 minutes.

- Hard caps. Daily (default 10) and monthly (default 100) message limits per recipient; extra messages are dropped, never queued. A warning arrives at 80% of a cap.

- No noise. No 'resolved' messages on WhatsApp; a wide outage is one message, repeated at most every 6 hours.

- Safe start. It begins in dry-run (logs only, no cost) and is switched on only after a review week.

6. Reading and acting on alerts

One failure produces one message. The header names the affected object and the worst severity; the lines below list each alert on it. Symptoms are held back while their cause is firing (a down link silences its signal, loss and error alerts), so the first message is the cause.

| Level | Meaning | Expected response |
| --- | --- | --- |
| Critical (red) | Something is down or about to stop working. | Act immediately. Reminders every 2 hours until fixed. |
| Warning (orange) | Degraded but still working. | Investigate within the working day. Reminders every 12 hours. |
| Capacity (08:00 digest) | A sector is over its client or throughput budget. | Plan an upgrade or rebalance. |

Common alerts and the first response

| Alert | What it means | First response |
| --- | --- | --- |
| Link down / link down prolonged | A backhaul link is unreachable (Telegram at 5 min; WhatsApp at 30 min). | Check power at both ends; if a whole POP is affected, check the POP router. |
| Weak signal / low quality / packet loss / high latency | The link is up but degraded for most of the last 30 minutes. | Look at the link's trend; suspect alignment, interference or weather. |
| Core router down | A router does not answer. | Check power and uplink. If a monitoring alert fires at the same time, the router is probably fine. |
| Sector down / customer-radio alerts | A sector or its customer radios are affected. | Check the sector's power and management route; open Sector Detail. |
| Subscriber mass outage | Online subscribers dropped sharply on a POP or router. | Check the POP router and backhaul first; one cause usually explains it. |
| Anomaly alerts | A measurement has drifted from its own recent normal. | Look for slow bleeds: eroding subscriber counts, router load creeping up. |
| Monitoring lost its network | The monitoring server itself is cut off. | Not a network outage. Device alerts are held back until it recovers. |
| Monitoring component down / delivery failing | Part of the platform or the alert path is impaired. | The platform owner restores it; devices behind it are usually fine. |
| Heartbeat 'down' (from outside) | The whole platform has gone silent. | Check the server is powered and online; inform the platform owner. |

Planned work: the affected links or routers are silenced for the duration, so maintenance does not page anyone; silences expire on their own.

7. What the NOC sees

| Dashboard | Answers |
| --- | --- |
| NOC Overview | What is wrong right now, where, and for how long? |
| PTP Link Monitoring | How healthy is each backhaul link, and is it degrading? |
| Router Fleet / Router Detail / WAN-LAN | Are routers up, loaded, and are their uplinks carrying traffic? |
| Sector Fleet / Sector Detail | Which sectors are overloaded or noisy, and which customer radios are weakest? |
| Subscriber Overview | How many customers are online vs provisioned, and who keeps dropping? |
| Capacity Planning | Where are we running out of capacity at busy hour? |
| Server Health | Is the monitoring platform itself healthy? |

NOC staff use read-only Viewer accounts.

8. Server health and troubleshooting

The platform watches its own server and components and reports problems with them separately from network alerts, so a monitoring fault is never mistaken for a network outage.

| What the platform checks on itself | Alerted when |
| --- | --- |
| Disk | Almost full, or will be full within 24 hours. |
| Memory and CPU | Memory runs low or CPU stays saturated. |
| Clock | The server's time drifts (timestamps and alert timing depend on it). |
| Every component | Any collector, the dashboard service or the alert router stops. |
| Alert rules | Rules fail to evaluate. |
| Alert delivery | Telegram or WhatsApp messages fail to send. |
| The server as a whole | The outside heartbeat stops (section 5). |

Common situations

| What you see | Usually means | What happens next |
| --- | --- | --- |
| Many devices 'down' at the same moment | A collector stopped, or the server lost its own network - not a real outage. | One monitoring alert says which; device alerts are held back until it is fixed. |
| No alert messages for a long time | Either all is well, or the alert path is broken. | The heartbeat and the delivery-failure alert catch a broken path; the Server Health dashboard confirms. |
| A newly added device shows as down | Wrong address, access settings, firewall, or (for sectors) the management route is missing. | Platform owner checks reachability and the device's monitoring access. |
| Dashboards empty | The metrics engine restarted or cannot collect. | Platform owner restores collection; history is kept. |
| Server disk filling | History growing. | Alerted 24 hours ahead; storage is extended or retention adjusted. |
| WhatsApp page not received | Dry-run mode, daily/monthly cap reached, or Meta account issue. | Visible on the Server Health budget panel; Telegram still carries every alert. |

9. Security

- Hosted in-house. One server inside the ISP's management network, with no leg in the customer network. No network data leaves the company except alert text sent to Telegram/WhatsApp.

- Nothing exposed to the internet. It only makes outgoing connections (alert delivery and the heartbeat).

- Read-only. It cannot change any router, radio or customer setting.

- SNMPv3 only. Encrypted, authenticated polling is the policy; compliant on the core router and SAF links, the rest migrating; AirFiber awaits a decision (section 2).

- Management separated from customers. Management addresses are not reachable from the customer side, sectors carry narrow routes only, and no routing protocols run on sectors (section 4).

- Restricted access. Dashboards for NOC workstations only; administrative interfaces for engineers only; internal collector ports are closed to the network.

- Credentials protected. Passwords and tokens are stored separately from the configuration, never in the code repository; an automated secret scanner checks every change.

10. Reliability

- Restarts on its own after a power cut or reboot.

- Watches itself (section 8), and an outside heartbeat covers total failure.

- Tested changes. Automated tests check the alert behaviour; the published platform image will not build if any test fails.

- Recoverable. The full configuration is version-controlled; a replacement server can be brought up from it, with credentials restored from an encrypted backup.

11. Running costs and hardware

| Item | Cost |
| --- | --- |
| Software | None - free, open-source components. |
| Telegram alerts | Free. |
| Independent heartbeat | Free tier(At the moment) |
| WhatsApp paging (optional) | Per message, billed by Meta; capped, so the worst-case monthly cost is fixed in advance. |
| Server | One dedicated Linux server (below) |

Recommended server for a 10,000-subscriber network (about 450-500 thousand tracked measurements, ~2 GB of history per day):

|  | Minimum | Recommended |
| --- | --- | --- |
| Processor | 4 cores | 8 cores |
| Memory | 16 GB | 32 GB |
| Storage | 500 GB SSD | 1 TB NVMe SSD |
| Operating system | Ubuntu Server LTS | Ubuntu Server LTS |

A Windows PC is not recommended as the production monitor: it needs a logged-in user, sleeps and reboots for updates, and would leave the network unmonitored during those windows.

12. Rollout status

| Milestone | Status |
| --- | --- |
| Platform built and tested (alerts, dashboards, subscriber intelligence) | Done |
| Pilot on live equipment (core router, backhaul links, sector) | Done |
| SNMPv3-only policy | Core router and SAF links compliant; sectors and other routers next; AirFiber decision pending |
| Telegram NOC group, strong admin password | Done |
| Independent heartbeat with Telegram notification | Done and tested |
| Production server procured and installed | Pending |
| Sector routes, POP by POP | Pending - starting with the pilot POP |
| Full device inventory loaded | Pending - with the production server |
| One-week settle-in on production, then optional WhatsApp paging | Pending |

13. What the business provides

- A dedicated server meeting the specification in section 11, on the management network, with uninterrupted power.

- Engineer time for the sector change (section 4): routes, SNMPv3 and access lists on each sector; route and customer-to-management firewall rule on each POP router.

- A decision on AirFiber monitoring under the SNMPv3-only policy (section 2).

- Network access from the server to all routers, links and sectors, and read-only monitoring access on each device.

- Agreement on who is on call, and which numbers (if any) receive WhatsApp pages.

- For WhatsApp paging: a registered business number and billing on Meta, and an agreed monthly cap.

- Named NOC staff for dashboard accounts.

14. Known limitations

- AirFiber cannot meet the SNMPv3-only policy (decision in section 2); airMAX support depends on firmware.

- Until a POP's sector routes are in, its sectors are reachable only through romon, without per-sector access restriction (section 4).

- Alerts are pushed through Telegram and WhatsApp; if both services were down at once, alerts would remain visible on the dashboards but not pushed.

- Customer radios are monitored through MikroTik sectors; radios on other sector types would need additional work.

15. Possible next phases

- Router and sector logs searchable beside the graphs.

- Plain-language incident summaries and natural-language questions about the network (AI-assisted).

- Recommended fixes for recurring problems, applied only with human approval.

- Integration with the billing / provisioning system.

16. Ownership

The platform is designed, built and operated by Valentine Chiemerie Ezechukwu, who owns its configuration, tuning, upgrades and the operational procedures. Requests for new alerts, dashboards, devices or reports go through the platform owner.